# Preliminary HAZOP Study Report — Node CDN-N02

Preflash Column Feed-Heating / Steam-Condensate Circuit (Engineer markup: “Node 23-02”)

PTT Phenol (PPCL) — CDN Section HAZOP | Generated by Process Expert Assistant (preliminary, for team review)

2026-06-17

Table of Contents

# PRELIMINARY HAZOP STUDY REPORT

## Node CDN-N02 — Preflash Column Feed-Heating / Steam-Condensate Circuit

| Plant | PTT Phenol Train II (PPCL), Map Ta Phut, Thailand |
| --- | --- |
| Section / Unit | CDN — Concentration, Decomposition, Neutralization (Concentration sub-section) |
| Node | CDN-N02 (engineer P&ID markup label: “Node 23-02”) |
| P&ID drawings | 14780-8120-25-23-0005, -0005A (primary); -0004 and -0007 (boundary crossings) |
| Licensor / Engineer | UOP (Honeywell) / POSCO Engineering |
| Report status | PRELIMINARY — DESKTOP FIRST-PASS (not a facilitated, team-validated result) |
| Revision / Date | Rev. A (Preliminary) — 2026-06-17 |
| Risk basis | PTT GC RAM W-(Q-MP)-002 R2; Economic category = BU (confirmed for PPCL) |

⚠️ STATUS & LIMITATION. This report is a machine-generated, wiki-grounded preliminary HAZOP first-pass prepared to seed the facilitated workshop. Causes, consequences, safeguard adequacy, risk rankings and recommendations are provisional and must be reviewed and finalized by the qualified HAZOP team (Leader, Scribe, Process/Operations/Instrument/Safety/Maintenance) per GC procedure P-(Q-MP)-OEMS-005. It is not an approved HAZOP record.

## 1. Introduction and Purpose

This node covers the feed-heating train of the Preflash Column (V-2301) plus its steam-condensate subsystem in the CDN Concentration sub-section. Its design purpose is to raise the Cumene Hydroperoxide (CHP)-containing oxidate feed from the Oxidation Section to the target Preflash Column feed temperature — recovering heat first, then trimming with steam — before the feed enters V-2301 for vacuum concentration of cumene.

The study was conducted as a desktop pass against the persistent CDN process-safety knowledge base (P&IDs, equipment data sheets, C&E table, SDS, and licensor operating manual) using the PTT GC HAZOP methodology (SG-(Q-MP)-014 R3) and risk matrix (W-(Q-MP)-002 R2).

## 2. Scope and Node Definition

### 2.1 Equipment within the node

| Tag | Description |
| --- | --- |
| E-2302A/B | Feed-Oxidate Exchangers (2 × 52.5% parallel) — preheat oxidate against hot OXI recirculate |
| E-2303 | Preflash Column Steam Heater — final SC1.5 steam trim heat |
| D-2308 | Preflash Column Steam Heater Condensate Drum |
| P-2308A/B | Steam Heater Condensate Pumps (sealless mag-drive) |

### 2.2 Node boundaries (engineer-confirmed 2026-06-17)

Inlet: - Process (cold feed): oxidate to E-2302A/B tube side — handover from Node 23-01 (feed filters X-2302A/B). - Process (hot side): recirculating oxidate to E-2302A/B shell (12”-CUL-23-011004) from OXI Oxidizer No.2 pumps. - Utility: SC1.5 steam to E-2303 tube side via SIS isolation valves UXV-0501/0502.

Outlet: - Process: heated oxidate to V-2301 Preflash Column (12”-CUL-23-005001) — handover to Node 23-05. - Process (hot side): shell-side recirculate return (16”-CUL-23-005001) to OXI. - Utility: steam condensate from P-2308A/B (4”-SCL.5-23-005006) to condensate system 66-0056.

Boundary interpretation (including the N01↔N02 inlet tie-in and the Dwg 0007 segment) was confirmed correct by the study owner on 2026-06-17.

## 3. Methodology and Governing Documents

- Method: GC 9-step HAZOP, parameter-first, full-record approach — SG-(Q-MP)-014 R3.

- Risk Assessment Matrix: W-(Q-MP)-002 R2 (5×5 PEES). Severity is fixed between initial and mitigated assessment; only Likelihood changes with safeguards.

- Economic severity category: BU (confirmed for PPCL, 2026-06-17) — GC ePHA Template v5.0 BU thresholds: Extreme(5) ≥100 M THB; High(4) 10–<100 M; Medium(3) 1–<10 M; Low(2) 0.1–<1 M; Very Low(1) <0.1 M.

- IPL credit: SG-(Q-MP)-014 Tables 6.4–6.6. SIL 1 = −1 likelihood level; SIL 2 = −2. Active fire protection / emergency response are not credited as IPLs.

- Anti-Bias: No previous Phenol/CDN HAZOP report was referenced. (The unrelated O-P3 example in raw/hazop/example/ was used only as a formatting reference and is never cited as content.)

- Acceptable risk (GC): Low or Very Low. Mitigated risk ≥ Medium requires a recommendation.

## 4. Design Intent and Normal Operating Parameters

The process side carries single-phase liquid oxidate (~22.6 wt% CHP); the utility side carries SC1.5 saturated steam → condensate. Heat is first recovered in E-2302A/B (oxidate-vs-oxidate), then trimmed in E-2303 with steam under feed-forward control (FIC-0501) from OXI flow/temperature.

| Tag / side | Design condition | Operating condition |
| --- | --- | --- |
| E-2302A/B tube (oxidate, CHP) | 12 kg/cm²g / FV @ 83→120 °C | ~82–83 °C |
| E-2302A/B shell (hot OXI recirc.) | 3.5 kg/cm²g / FV @ 195/250 °C | hot recirculate from OXI |
| E-2303 shell (oxidate, CHP) | 3.5 kg/cm²g / FV @ 195/250 °C | in ~82 °C → out ~83 °C to V-2301 |
| E-2303 tube (SC1.5 steam) | 7 kg/cm²g / FV @ 120/195 °C | SC1.5 steam ~120–133 °C sat. |
| D-2308 (condensate) | 7 kg/cm²g / FV @ 195 °C | 0.9 kg/cm²g / 117 °C; NLL 550 mm |
| P-2308A/B (condensate) | 18.0 m³/h @ 1.53 kg/cm² ΔP | running/standby; FY-0502 auto-start |

Data conflict (action R-004): the unit page lists oxidate feed (S229) pressure as 78 kg/cm²G, inconsistent with E-2302 tube design (12) and X-2302 filter design (8). Treated as a probable transcription error; feed-line design pressure / piping class to be confirmed.

## 5. Governing Hazard — Cumene Hydroperoxide (CHP)

CHP is the highest-hazard chemical in the plant: an organic peroxide (Type E) with decomposition onset ≈ 80 °C (worst-case PSS basis; SADT 60–80 °C), undergoing autocatalytic, strongly exothermic decomposition that releases heat and oxygen and can deflagrate/detonate when concentrated. In this node the oxidate is already ~82–83 °C and the heating medium is ~120–133 °C — so over-temperature of the CHP-containing oxidate is the dominant hazard. Iron/transition-metal and acid contamination are potent decomposition catalysts (304L SS wetted parts are used throughout; no copper alloys permitted).

## 6. HAZOP Worksheet (Deviation Analysis)

Risk ranking key: RR after the arrow is the mitigated (with existing safeguards) risk. L = likelihood (1–5); severity shown as P/En/Ec/S. IL/ESD = SIS trip/ESD safeguard.

### 6.1 Flow — No / Low Flow

Cause 1.1 — Loss of oxidate feed (X-2302 filter plugging, OXI feed-pump trip, or spurious UXV-0401 closure) with SC1.5 steam still on. - Consequence: stagnant CHP oxidate overheats in E-2303/E-2302 above 80 °C → autocatalytic decomposition → heat + O₂ → overpressure, exchanger/line rupture → LOPC of hot CHP → fire/explosion, potential multiple fatalities. - Initial: L4 — 5/4/5/4 — Extreme - Safeguards: FXSLL-0401A/B/C (2oo3, SIL 1) → UC-2301 closes UXV-0401 feed and UXV-0501/0502 steam (IL/ESD, IPL=1); TXSHH-0502A/B (1oo2, SIL 1) backup overheat trip (IL/ESD, IPL=1); PDAH-0301 filter ΔP alarm + operator (IPL=1). - Mitigated: L2 — 5/4/5/4 — Medium → R-001

Cause 1.2 — Spurious UC-2301 trip: operability only (steam also isolated — safe direction). Low, no recommendation.

### 6.2 Flow — More Flow

Cause 2.1 — OXI throughput increase / feed control upset → higher oxidate flow. - Consequence: under-heating → colder feed to V-2301 → poorer flash, off-spec concentration; no over-temperature hazard (safe direction). Production impact only. - Initial: L3 — 1/1/2/1 — Low (Ec: BU, est. 0.1–<1 M THB → Ec 2) - Safeguards: FIC-0501 feed-forward steam control; downstream V-2301 controls. - Mitigated: L2 — 1/1/2/1 — Low → none.

### 6.3 Flow — Reverse Flow

N/A — not credible: feed delivered under positive pump pressure from OXI; no credible reversing source within node.

### 6.4 Temperature — More / High Temperature ★ dominant node hazard

Cause 4.1 — E-2303 steam control fails to max heat (FIC-0501/FXY-0501 malfunction or steam valve fails open). - Consequence: oxidate heated above 80 °C → autocatalytic CHP decomposition in E-2303 and downstream → overpressure, rupture, LOPC → fire/explosion, potential multiple fatalities. - Initial: L4 — 5/4/5/4 — Extreme - Safeguards: TXSHH-0502A/B (1oo2, SIL 1) → UC-2301 closes UXV-0501/0502 (IL/ESD, IPL=1); TXAHN-0502 DCS critical alarm + operator (IPL=1); UXV-0501+0502 redundant series steam isolation. - Mitigated: L2 — 5/4/5/4 — Medium → R-001

Cause 4.2 — E-2302A/B hot OXI recirculate abnormally hot (upstream OXI temperature excursion → over-preheat). - Consequence: same CHP over-temperature / decomposition chain, initiated on the preheat side. - Initial: L3 — 5/4/5/4 — High - Safeguards: TXSHH-0501 (1oo1, SIL 1) → UC-2301 ESD (IL/ESD, IPL=1); TXAHN-0501 alarm + operator (IPL=1); OXI cross-trip UC-2201→UC-2301 (IL/ESD, IPL=1). - Mitigated: L1 — 5/4/5/4 — Low → setpoint margin confirmed under R-001.

### 6.5 Temperature — Less / Low Temperature

Cause 5.1 — Loss of SC1.5 steam or loss of E-2302 preheat. - Consequence: cold feed to V-2301 → poor flash, off-spec, column upset, production loss. Safe direction for CHP. - Initial: L4 — 1/1/2/1 — Medium (Ec: BU est. 0.1–<1 M THB → Ec 2; team to validate vs rate-loss financials) - Safeguards: temperature indication/control (TI/TT-0505); downstream V-2301 controls/analyzers. - Mitigated: L3 — 1/1/2/1 — Low → none (Economic estimate to be validated).

### 6.6 Pressure — More / High Pressure

Cause 6.1 — Process (CHP) side blocked in with heat source (e.g., isolated for maintenance with residual CHP; fire; or decomposition gas) → liquid-full thermal/decomposition overpressure. - Consequence: overpressure beyond E-2302 tube (12) / E-2303 shell (3.5 FV) → rupture → LOPC of CHP → fire/explosion. - Initial: L3 — 5/4/5/4 — High - Safeguards: in normal lineup relieves to V-2301 via PSV-23-0401A/B/C/D (DIERS runaway case, IPL=2); high-temp SIFs cut heat source (IL/ESD, IPL=1). Gap: no dedicated PSV on E-2302A/B / E-2303 process side once block-isolated from V-2301 relief. - Mitigated: L2 — 5/4/5/4 — Medium → R-003

Cause 6.2 — SC1.5 steam supply overpressure into E-2303 tube / condensate circuit. - Consequence: steam-side overpressure (low chemical hazard) with possible escalation to process side via tube failure (§6.11). - Initial: L3 — 2/1/2/1 — Low - Safeguards: PSV-23-0501 on D-2308 (set 7.0 kg/cm²g, IPL=2). - Mitigated: L1 — 2/1/2/1 — Very Low → confirm relief path covers steam-supply overpressure (note under R-003).

### 6.7 Pressure — Less / Vacuum

Cause 7.1 — Steam-side vacuum on trip (sudden UXV-0501/0502 closure collapses steam → vacuum / water hammer). - Consequence: vacuum collapse / water hammer in E-2303 tubes → mechanical damage → tube leak (§6.11). - Initial: L3 — 2/1/2/1 — Low - Safeguards: E-2303 tube and D-2308 rated for Full Vacuum (inherent design). - Mitigated: L2 — 2/1/2/1 — Low → confirm no vacuum-breaker requirement given FV design.

### 6.8 Level — More / High Level (D-2308)

Cause 8.1 — LIC-0502/LV-0502 condensate return fails closed / return header backpressure. - Consequence: condensate backs into E-2303 tubes → flooding → loss of heat transfer → low/erratic feed temperature (links §6.5); possible water hammer. - Initial: L4 — 2/1/2/1 — Medium - Safeguards: LAH-0502 alarm + operator (IPL=1); LIC-0502 control; HLL 740 mm. - Mitigated: L3 — 2/1/2/1 — Low → none.

### 6.9 Level — Less / Low Level (D-2308)

Cause 9.1 — Excess pump-out / loss of condensate make / LV-0502 fails open. - Consequence: P-2308A/B lose NPSH → cavitation / steam blow-through to condensate system; pump damage. - Initial: L4 — 2/1/2/1 — Medium - Safeguards: LAL-0502 alarm + operator (IPL=1); LLL 260 mm; sealless mag-drive (no seal leak path). - Mitigated: L3 — 2/1/2/1 — Low → none (team to consider LALL/auto-stop to protect P-2308).

### 6.10 Service Failures

Cause 10.1 — Loss of power → both P-2308A/B stop (Type L, not reliable power): condensate accumulates → backs into E-2303 (as §6.8); no direct CHP hazard. L4 — 2/1/2/1 Medium → L3 Low with LAH-0502.

Cause 10.2 — Loss of instrument air → control valves to fail position: if SC1.5 steam valve fails open → High Temperature (§6.4); if fails closed → Low Temperature (§6.5). Action: confirm fail-safe action of the steam control valve and LV-0502 (control-valve register).

Cause 10.3 — Loss of SC1.5 steam: covered under §6.5.

### 6.11 Composition / As Well As — Tube Leak / Rupture

Cause 11.1 — E-2303 tube leak/rupture (SC1.5 steam ↔ oxidate/CHP). - Consequence: process oxidate (~8 kg/cm²g) leaks to the lower-pressure steam-condensate side → CHP migrates to the steam-condensate system (D-2308 → return 66-0056), which is not rated or monitored for CHP → uncontrolled CHP in a utility system, off-site migration, potential decomposition/fire; conversely water/steam ingress contaminates the CHP oxidate. - Initial: L3 — 4/3/4/3 — High - Safeguards: 304L SS tubes; construction-stage helium leak test (one-time, IPL=0); periodic inspection (non-IPL). No on-line CHP/HC detection on D-2308 condensate identified. - Mitigated: L3 — 4/3/4/3 — High → R-002

Cause 11.2 — E-2302A/B tube leak/rupture (HP oxidate tube ↔ LP OXI recirculate shell). - Consequence: HP→LP leak → shell overpressure; both fluids CHP-containing (lower contamination concern). Relief via shell/column path. - Initial: L3 — 3/3/3/2 — Medium - Safeguards: 304L SS tubes; shell relief path; high-temp SIFs limit escalation. - Mitigated: L2 — 3/3/3/2 — Low → confirm shell-side relief adequacy under R-003.

### 6.12 Composition — Other Than (Contamination)

Cause 12.1 — Iron/transition-metal or acid contamination of CHP oxidate (catalysts for decomposition). - Consequence: catalysed CHP decomposition at lower temperature than expected → localized exotherm → as §6.4. - Initial: L2 — 5/4/5/4 — Medium - Safeguards: 304L SS wetted parts (no Cu/Cu-alloy, no carbon steel in CHP service); upstream X-2302 filtration; high-temp SIFs (IPL=1, inherent material selection). - Mitigated: L1 — 5/4/5/4 — Low → confirm condensate/contamination monitoring under R-002.

### 6.13 “Other” Checklist Summary

| Topic | Preliminary finding |
| --- | --- |
| Relief scenario | Block-isolated maintenance case not covered by dedicated PSV → R-003; confirm D-2308 PSV-23-0501 bounds steam-supply overpressure. |
| Instrumentation | Two independent SIL 1 overheat SIFs — adequacy vs S5 to be LOPA-confirmed (R-001); confirm setpoints < 80 °C onset with margin. |
| Sampling | No process sample point inside node boundaries — confirm adequacy with team. |
| Corrosion | CHP + Fe catalysis — 304L SS confirmed; KCS acceptable on condensate side. |
| Service failure | Confirm steam control valve fail-action and P-2308 power classification. |
| Maintenance | Confirm safe CHP drain/purge + relief during single-train isolation (R-003). |
| Start-up | HXS-0102 bypasses FXSLL-0401 during startup — confirm TXSHH-0502 overheat protection stays active (R-001). |
| Human factor | Feed-forward control complexity; alarm response time vs CHP onset (R-001). |

## 7. Recommendations (Preliminary)

| Rec# | Risk | Discipline | Recommendation |
| --- | --- | --- | --- |
| R-001 | Medium | Instrument / Process | Perform LOPA / SIF verification confirming the combined overheat protection (TXSHH-0501 SIL 1 + TXSHH-0502A/B SIL 1 + redundant UXV-0501/0502) achieves tolerable risk for the S5 CHP-decomposition consequence; confirm all TXSHH setpoints sit below the 80 °C onset with adequate margin; confirm startup bypass HXS-0102 does not disable temperature protection. |
| R-002 | High | Process / Instrument | Provide on-line detection of CHP/hydrocarbon carryover into the steam-condensate system (conductivity/pH or HC analyzer with alarm and/or condensate divert) at D-2308 / condensate return, to detect an E-2303 tube leak and prevent CHP migration to the unrated utility condensate system. |
| R-003 | Medium | Process / Mechanical | Review overpressure/thermal-relief protection for E-2302A/B and E-2303 process (CHP) side when block-isolated for maintenance (no dedicated PSV once isolated from V-2301 relief); confirm safe CHP draining/purging for single-train isolation. |
| R-004 | Low | Process | Resolve the oxidate feed pressure data conflict (S229 78 kg/cm²G vs E-2302 tube 12 / X-2302 8 kg/cm²g) and confirm feed-line design pressure / piping class. |

## 8. Interlock / ESD Safeguards Credited in this Node

All are existing UC-2301 (Concentration Section) ESD functions; cross-checked against the CDN Cause & Effect table (Causes 1/2/3).

| Safeguard | Voting / SIL | C&E Cause | Final element / action |
| --- | --- | --- | --- |
| FXSLL-0401A/B/C | 2oo3 / SIL 1 | Cause 1 | Close UXV-0401 (feed) + UXV-0501/0502 (SC1.5 steam) |
| TXSHH-0501 | 1oo1 / SIL 1 | Cause 2 | UC-2301 Concentration ESD |
| TXSHH-0502A/B | 1oo2 / SIL 1 | Cause 3 | Close UXV-0501/0502 (SC1.5 steam) |
| UXV-0501 + UXV-0502 | — (final elements) | — | Redundant series SC1.5 steam isolation |

## 9. Outstanding Items, Assumptions and Data Conflicts

- Preliminary status — all entries require HAZOP team validation; HAZOP Coordinator sign-off on P&ID Readiness (Table A6.2-3) and PSV set-pressures for V-2301/V-2302 remain open per study-info.

- Economic estimates (#6.2/6.5) are first-pass BU values — validate against actual production-loss financials.

- Feed pressure data conflict (R-004).

- Control-valve fail actions (steam valve, LV-0502) to be confirmed from the control-valve register.

- Markup record — recommend filing the “Node 23-02.pdf” markup in raw/pid/ for the document trail.

## 10. Limitation & Disclaimer

This preliminary report was generated from the wiki knowledge base as an aid to the HAZOP workshop. It does not replace the qualified, multi-disciplinary HAZOP team’s judgment, the facilitated session, or the formal report-approval and action close-out workflow defined in GC procedure P-(Q-MP)-OEMS-005. Risk rankings are provisional and shall not be used as the basis for design or operating decisions until reviewed, amended as required, and approved by the HAZOP Leader.

## Appendix — References

- Risk matrix: W-(Q-MP)-002 R2 (Economic = BU for PPCL)

- Methodology: SG-(Q-MP)-014 R3 (9-step, guidewords, IPL Tables 6.4–6.6)

- Procedure: P-(Q-MP)-OEMS-005 R4

- Cause & Effect: 14780-8120-25-23-0002 (UC-2301)

- Equipment data sheets: E-2302, E-2303, D-2308, P-2308 (Rev Z1 As-Built)

- CHP hazard basis: SDS 80-15-9 (decomposition onset ≈ 80 °C)

- Source node worksheet (live): wiki/hazop/nodes/cdn-N02.md